Intel ® Celeron® J1900 Quad Core Processor Mini PC

Nano N3

Features：

◇ All aluminum structure，fanless design；

◇ Onboard Intel Celeron J1900 or N3540 quad core processor Optional；

◇ Max. Support DDR3L 8GB RAM；

◇ Support 1*MSATA，1*2.5” HDD；

◇ Support 2*1000M LAN；

◇ Support OS：

| Specification |  |
| --- | --- |
| Model | Nano N3 |
| Color | Black (Color can be customized) |
| Material | High-quality all aluminum，surface anodic oxidation |
| Processor | Onboard Intel Celeron J1900（Quad core quad threading，2MB Second Level Cache 1.99GHz, pulse frequency 2.42GHz） |
|  | Onboard Intel Pentium N3540（Quad core quad threading，2MB cache，2..16GHz，pulse frequency 2.66GHz） |
| Chipset | J1900 or N3540（Single Chip) |
| RAM | Support 1*DDR3L 1333/1600MHz SODIMMRAM slot，Max. Support 8GB |
| BIOS | AMI UEFI BIOS |
| Display Chip | Integrated Intel HD Graphics |
| Display Port | 1*VGA，1*HDMI（Support synchronous and asynchronous dual display） |
| Front I/O | 1*Power Switch，1*USB3.0，5*USB2.0，1*RS232 COM |
| Rear I/O | 1*Audio，2*RJ45 LAN，1*VGA，1*HDMI，1*DC Input |
| Expansion Slot | Support 1*MINI PCIE Slot（WIFI and Bluetooth Optional） |
|  | Support 1*MINI PCIE Slot（Support MSATA） |
| Network | 2*10/100/1000M RTL8111E 1000M LAN |
| Storage | 1*MSATA，1*2.5” HDD |
| Input Voltage | Support DC 12V Input |
| Other Function | Auto power on when there is electricity、Timing boot、Wake on LAN、PXE boot、Watchdog（0～255 level） |
| Work Temp | 0℃ ～ +50℃（Commercial HDD），-20℃ ～ +50℃（Industrial SSD），Surface air flow |
| Work Humidity | 5% ～ 95% non-condensing |
| Certificates | CE、 CCC、 FCC Class A、 ROHS |
| Dimension | 134 * 126 * 38.5 mm |
| Net Weight | 0.7kg |
| Application | Industrial Automation, Medical, Logistics Transportation, Warehouse, Electronic Education etc. |

Nano N3 Drawing

| Accessories List |  |  |
| --- | --- | --- |
| Name | Quantity | Specification |
| Power Adapter | 1 | 12V4A |
| Power Cord | 1 | Any Plug Optional (China, US, EU, UK etc.) |
| VESA Bracket | 1 | Standard |

| Order Information |  |  |  |
| --- | --- | --- | --- |
| Configuration 1 | Nano N3 | Intel Celeron J1900 | Standard Port，6*USB，1*COM |
| Configuration 1 |  | Intel Pentium N3540 | Standard Port，6*USB，1*COM |